13.5 Extracting of Fe

Name: ____________________    Class: ____________________  Due date: 6.5

1. Impure iron from the blast furnace is converted to steel as shown.

Which statement about the process is correct?								（   ）

A Acidic oxides are added to remove alkaline impurities.

B Coke is added as a reducing agent.

C Oxygen is blown in to oxidise the impure iron.

D The steel produced contains less carbon than the impure iron.

2. Iron from a blast furnace is treated with oxygen and with calcium oxide to make steel.

Which substances in the iron are removed?								（   ）

|  | oxygen removes | calcium oxide removes |
| --- | --- | --- |
| A | carbon | acidic oxides |
| B | carbon | basic oxides |
| C | iron | acidic oxides |
| D | iron | basic oxides |

3. Four reactions that take place in the blast furnace to produce iron are shown.

Which reaction is used to keep the furnace hot?						（   ）

A C+O2→CO2

B CO2+C→2CO

C Fe2O3+3C→2Fe+3CO

D Fe2O3+3CO→2Fe+3CO2

4. Iron is extracted from hematite in the Blast Furnace.

The hematite contains silica as an impurity.

What reacts with this impurity to remove it?								（   ）

A calcium oxide 		B carbon 		C carbon dioxide 	D oxygen

5. The diagram shows a blast furnace.

In which part is iron ore changed to iron?								（   ）

6. Which statement is incorrect?											（   ）

A Carbon dioxide is a waste product in the extraction of iron.

B Carbon monoxide is a reducing agent.

C The extraction of iron from hematite involves reduction.

D When iron is converted into steel, oxygen is used to oxidise the iron.

7. Equations P and Q represent two reactions which occur inside a blast furnace.

&P Fe2O3+3CO→2Fe+3CO2&Q CaCO3→CaO+CO2

Which type of reactions are P and Q?										（   ）

|  | P | Q |
| --- | --- | --- |
| A | redox | redox |
| B | redox | thermal decomposition |
| C | thermal decomposition | redox |
| D | thermal decomposition | thermal decomposition |

8. Which substance is not involved in the extraction of iron from hematite?	（   ）

A carbon 	B carbon monoxide 	C calcium carbonate 	D nitrogen

9. Iron is extracted from its ore in a Blast Furnace.

Hematite, coke, limestone and hot air are added to the furnace.

Which explanation is not correct?											（   ）

A Coke burns and produces a high temperature.

B Hematite is the ore containing the iron as iron oxide.

C Hot air provides the oxygen for the burning.

D Limestone reduces the iron oxide to iron.

10. Iron is extracted from its ore, hematite, in the blast furnace.

- The temperature inside the blast furnace can rise to 2000∘C

- Write an equation for the exothermic reaction which causes this high temperature.

.......................................................................................................................................

...................................................................................................................................[1]

- Carbon monoxide is formed in the blast furnace. This reduces the ore hematite,Fe2O3 to iron.

- Explain how carbon monoxide is formed in the blast furnace.

- .......................................................................................................................................................................................................................................................................... [2]

- Write an equation for the reduction of hematite by carbon monoxide.

- ...................................................................................................................................[2]

- Explain why it is necessary to add limestone, calcium carbonate, to the blast furnace. Include an equation in your explanation.

- Most of the iron from the blast furnace is converted into mild steel.

- A method of preventing the steel from rusting is coating it with zinc.

- What is the name of this method of rust prevention?

- ..................................................................................................................................[1]

- Explain, using the idea of electron transfer, why zinc- coated steel does not rust even when the coating is scratched and the steel is in contact with oxygen and water.

......................................................................................................................................

.......................................................................................................................................

...................................................................................................................................[3]