16.1 Petroleum

Name: ____________________    Class: ____________________  Due date: 1.13

1. Which method is used to obtain petrol from petroleum?				（   ）

A crystallisation 	B diffusion 		C filtration 		D fractional distillation

2. Which fraction from the fractional distillation of petroleum does not match its correct use?																（   ）

|  | fraction | use |
| --- | --- | --- |
| A | fuel oil | domestic heating |
| B | kerosene | jet fuel |
| C | naphtha | making roads |
| D | refinery gas | for heating and cooking |

3. Which properties of the different compounds in petroleum enable its separation into fractions?															（   ）

1 boiling point

2 chain length

3 chemical reactivity

4 solubility in water

A 1 and 2 				B 1 and 3 			C 2 and 4		  D 3 and 4

4. Bitumen is a substance obtained from the fractional distillation of petroleum.

Which row describes its boiling point and the size of its molecules?		（   ）

|  | boiling point | size of molecules |
| --- | --- | --- |
| A | high | large |
| B | high | small |
| C | low | large |
| D | low | small |

5. The table shows some fractions that are obtained from petroleum by fractional distillation, together with some of their uses.

| fraction | use |
| --- | --- |
| refinery gas | cooking |
| gasoline | fuel for cars |
| 1 | making chemicals |
| 2 | jet fuel |
| 3 | fuel for ships |
| bitumen | making roads |

Which row correctly identifies fractions 1, 2 and 3?						（   ）

|  | 1 | 2 | 3 |
| --- | --- | --- | --- |
| A | diesel oil | fuel oil | lubricating fraction |
| B | fuel oil | diesel oil | kerosene |
| C | kerosene | naphtha | diesel oil |
| D | naphtha | kerosene | fuel oil |

6. The table shows the composition of four different types of petroleum (crude oil).

| fraction | Arabian Heavy /% | Arabian Light /% | Iranian Heavy /% | North Sea /% |
| --- | --- | --- | --- | --- |
| gasoline | 18 | 21 | 21 | 23 |
| kerosene | 11.5 | 13 | 13 | 15 |
| diesel oil | 18 | 20 | 20 | 24 |
| fuel oil | 52.5 | 46 | 46 | 38 |

Which type of petroleum is best for the motor vehicle industry?				（   ）

A Arabian Heavy 		B Arabian Light 	C Iranian Heavy 	D North Sea

7. Which statement about petroleum is not correct?							（   ）

A It can be separated into useful substances by fractional distillation.

B It consists mainly of hydrocarbons.

C It is found underground in many parts of the world.

D Its main use is for making lubricants and polishes.

8. The diagram shows the separation of petroleum into fractions.

What could X, Y and Z represent?											（   ）

|  | X | Y | Z |
| --- | --- | --- | --- |
| A | diesel oil | lubricating fraction | paraffin |
| B | lubricating fraction | diesel oil | paraffin |
| C | paraffin | lubricating fraction | diesel oil |
| D | paraffin | diesel oil | lubricating fraction |

9. Petroleum is an important fossil fuel.

Which row correctly describes petroleum?									（   ）

|  | type of substance | composition |
| --- | --- | --- |
| A | compound | mainly hydrocarbons |
| B | compound | only hydrogen and carbon |
| C | mixture | mainly hydrocarbons |
| D | mixture | only hydrogen and carbon |

10. Petroleum is separated into fractions.

Which statement is not correct?											（   ）

A Each fraction contains a mixture of hydrocarbon molecules.

B Fuel oil burns easily and is used as fuel in cars.

C Refinery gas is the fraction containing the smallest molecules.

D The fractions are separated depending on their boiling point range.

11.. Some fractions obtained from petroleum are listed.

|  | fraction | use | position collected in the fractionating column |
| --- | --- | --- | --- |
| 1 | gasoline | waxes and polishes | below refinery gas |
| 2 | bitumen | making roads | above kerosene |
| 3 | kerosene | jet fuel | below gasoline |
| 4 | refinery gas | heating and cooking | above gasoline |

Which rows are correct?													（   ）

A 1,3 and 4 			B 2,3 and 4			 C 3 and 4 only			 D 4 only

12. Petroleum contains hydrocarbons which are separated by fractional distillation.

- Complete the following definition of a hydrocarbon.

- A hydrocarbon is a compound which .........................................................................[2]

- ii.Explain what is meant by the term fractional distillation.

- .....................................................................................................................................

- ...................................................................................................................................[2]

13.Petroleum is a source of many important chemicals.

- Name two industrial processes which must take place to produce alkenes from petroleum.

.......................................................................................................................................

...................................................................................................................................[2]

14.Petroleum is a mixture of hydrocarbons. It is separated into useful fractions by fractional distillation. This can be done using the fractionating column shown.

- What happens to the petroleum at point X, before it enters the fractionating column?

..................................................................................................................................[1]

- ii.State two ways in which fraction O differs from fraction L.

- ................................................................................................................................[2]